# Purpose

The purpose of this procedure is to ensure that the correct information for setting out the works is available, the works are set out accurately and that the records are properly maintained.

# Scope

The procedure covers the control of the original primary survey, the establishment of temporary control points and bench marks and any subsequent setting out including the recording of data.

# Explanation of terms

Primary survey:		This consists of the establishment of a network of principal control stations from which the works can be set out

Secondary survey:		This consists of the establishment of a series of secondary control points from which the day to day setting out and levelling can be carried out

Subsequent setting out	Day to day setting out and levelling from the secondary control points

# Interface documentation

Procedure Control of Setting out and Measuring Equipment, QU-P-04

# Responsibilities

The Project Manager is responsible for ensuring that this procedure is carried out. It shall be periodically audited by the Project Quality Engineer to ensure continued adherence. The Project Surveyor shall carry out the requirements of this procedure.

# Description

Prior to the carrying out of the Primary survey, the Project Engineering Manager shall obtain the necessary information about the base survey reference pints and levels from the client or from the official ordinance survey records as appropriate. He shall check the locations of these and verify that they are in accordance with the available maps or sketches. Any discrepancies in the information supplied shall be resolved prior to proceeding with the survey.

He shall draft a method statement outlining how the necessary primary survey pints are to be established, including any checks that will be performed to make sure that any transfer of coordinates and level is done to the accuracy required. The actual survey shall be recorded ant the primary survey points clearly identified on a drawing, duly signed by him with any verification checks by others initialled accordingly. If required by the contract the primary survey will be submitted to the Client for approval. A statement outlining the degree of accuracy of all survey activities shall be included in the method statement, or in a separate written instruction to the Surveyors.

The person/s responsible for carrying out the primary survey shall be nominated by the Project Manager in consultation with the Project Engineering Manager.

The location of secondary survey points will as far as possible be identified on the primary survey method statement.

Secondary surveys are carried out by the designated surveyors as the work progresses and their requirement is established. Records of these surveys shall be retained by the Surveyor concerned. Secondary survey points shall constitute the approved points for setting out the work.

All survey and setting out work shall be in accordance with the methods outlined in an approved publication on the topic. Any additional clarification or special instructions shall be issued to Surveyors by the Project Engineering Manager in the form of written notes. Records of all setting out shall be maintained by the person undertaking the work.

The Senior Surveyor shall carry out periodic checks on the work of the surveyors ensuring that the works are at all times set out in accordance with the contract requirements.

All observations shall be recorded in the Surveyor’s survey / level book listing:

- The name of the surveyor

- The date of the survey or setting out activities

- Description of the work section, start and finish points of the survey activity

- Identification of the instrument including instrument number

- Any calibration or accuracy checks carried out on the instrument

- All observations

- The results of any checks and observations

# Attachments

None